COMPUTER NETWORKS

Week 1 – Complete Study Notes

Based on: Computer Networks, 6th Edition – Tanenbaum & Wetherall

Instructor: Dr. Mohammad Shahidul Islam

# Topic 1: What is a Computer Network?

## Definition

A computer network connects two or more autonomous computers so they can communicate and share resources.

⭐ Important: Key word: Autonomous = Each computer works independently. It does NOT need the other computer to function.

## Simple Real-Life Example

Imagine your home. You have a laptop, a mobile phone, and a printer. When you connect them using Wi-Fi, they form a small computer network.

- Your phone can print a document using the shared printer.

- Your laptop can access files from the phone.

- All devices can share one internet connection.

🧠 Memory Trick: NETWORK = Devices + Cables/Wireless + Sharing. Think of it like a group of friends sharing notes!

🇧🇩 Bangla Summary: কম্পিউটার নেটওয়ার্ক হলো দুই বা তার বেশি কম্পিউটারের সংযোগ, যেখানে তারা তথ্য এবং রিসোর্স শেয়ার করতে পারে। প্রতিটি কম্পিউটার স্বাধীনভাবে কাজ করতে পারে।

# Topic 2: Types of Networks – LAN, MAN & WAN

Networks are classified by their geographical size. Think of it like: small area → big area.

## LAN – Local Area Network

- Covers a small area like a room, building, or campus.

- Example: The computer lab in your university is a LAN.

- Speed is very high (usually 100 Mbps to 10 Gbps).

- Owned and managed by one organization.

## MAN – Metropolitan Area Network

- Covers a city or town.

- Example: A cable TV network across Dhaka city is a MAN.

- Larger than LAN but smaller than WAN.

- Sometimes connects multiple LANs within a city.

## WAN – Wide Area Network

- Covers a country or even the entire world.

- Example: The Internet is the biggest WAN in the world.

- Speed is lower than LAN because data travels long distances.

- Uses telephone lines, satellites, and fiber optic cables.

| Network Type | Coverage Area / Example |
| --- | --- |
| LAN (Local Area Network) | Room, Building, Campus → e.g., University computer lab |
| MAN (Metropolitan Area Network) | City → e.g., Dhaka city cable TV network |
| WAN (Wide Area Network) | Country / World → e.g., The Internet |

🧠 Memory Trick: L = Local (small), M = Metro/City (medium), W = Wide (very large). Remember: L → M → W = Small → Medium → Large!

🇧🇩 Bangla Summary: LAN হলো ছোট এলাকার নেটওয়ার্ক (যেমন: বিশ্ববিদ্যালয়ের ল্যাব)। MAN হলো শহরের নেটওয়ার্ক। WAN হলো দেশ বা পৃথিবীজুড়ে বিস্তৃত নেটওয়ার্ক (যেমন: ইন্টারনেট)।

# Topic 3: Applications of Networks

Why do we use computer networks? Networks make life easier in many ways:

## 1. Resource Sharing

- Hardware sharing: Many computers share one printer or one hard disk.

- Software sharing: Many users use one application installed on a server.

Real-life example: In an office, 20 employees share one network printer. Nobody needs their own printer!

## 2. Information Sharing

- Files and databases can be accessed from anywhere.

- Google Search is a good example – you search for information from any location.

Real-life example: A hospital database where doctors from different floors access the same patient records.

## 3. Communication

- Email: Send messages to anyone in the world instantly.

- Message broadcast: Send one message to many people at once.

- Remote computing: Use a computer that is physically far away.

Real-life example: You send an email from Bangladesh to someone in USA. It reaches in seconds.

## 4. Distributed Processing (GRID Computing)

- A big task is divided among many computers.

- Each computer solves a small part of the problem.

Real-life example: Scientists use thousands of computers across the world to analyze weather patterns.

## Network Applications We Use Daily

- E-mail

- Web Sites (WWW – World Wide Web)

- E-Commerce (Online shopping – Daraz, Amazon)

- Internet Telephony (VoIP – WhatsApp calls, Zoom)

- Video Conferencing

- Instant Messengers (WhatsApp, Messenger)

- Internet Radio and Streaming

🧠 Memory Trick: Networks = Sharing + Communication + Information Access. Think: SHARE, TALK, FIND.

🇧🇩 Bangla Summary: নেটওয়ার্কের মাধ্যমে আমরা রিসোর্স শেয়ার করি (প্রিন্টার, সফটওয়্যার), তথ্য শেয়ার করি (ইমেইল, ওয়েবসাইট), এবং যোগাযোগ করি (VoIP, ভিডিও কল)। ইন্টারনেটও একটি বিশাল নেটওয়ার্ক।

# Topic 4: Network Topology (BSRM)

Topology means the layout or shape of the network. How are the computers and cables arranged?

⭐ Important: Memory trick for 4 topologies: B-S-R-M = Bus, Star, Ring, Mesh

## 1. Bus Topology

All devices are connected to ONE single cable called the 'bus' or 'backbone'.

- Data travels along the cable in both directions.

- Simple and cheap to set up.

- Problem: If the main cable breaks, the entire network fails.

- Used in older Ethernet networks.

Real-life example: Like a street with houses on both sides — the road is the bus cable!

## 2. Star Topology

All devices are connected to a central device called a Hub or Switch.

- Most popular topology used today in homes and offices.

- If one cable fails, only that one device is affected.

- If the central hub/switch fails, the entire network fails.

- Looks like the spokes of a bicycle wheel.

Real-life example: Your home Wi-Fi router is the center. Your phone, laptop, and TV all connect to it.

Extended Star / Tree Topology

- Larger networks use multiple stars connected together.

- Also called Tree Topology.

- Uses routers, switches, and bridges to filter traffic.

- More efficient – sends data only to the correct destination.

## 3. Ring Topology

All devices are connected in a circle. Data travels around the ring from one device to the next.

- A frame (small packet of data) travels around the ring.

- Each device checks if the data is for it. If yes, it takes it.

- Single Ring: Data flows in one direction only.

- Dual Ring: Data flows in both directions. More reliable.

Real-life example: Imagine children sitting in a circle passing a note. Each child reads the name on the note. If it is for them, they keep it. Otherwise, they pass it on.

## 4. Mesh Topology

Every device is connected to every other device. Multiple paths exist between devices.

- Very reliable – if one path fails, data takes another path.

- Used in WANs and mission-critical networks (banks, military).

- Very expensive to install because of many cables.

- Provides fault tolerance (network still works even if some links fail).

Real-life example: Highways between major cities — if one road is blocked, you take another route!

| Topology | Key Feature / Weakness |
| --- | --- |
| Bus | One cable for all. Cheap but single point of failure. |
| Star | Central hub. Easy to manage but hub failure = network down. |
| Ring | Circle connection. Data passes node to node. |
| Mesh | Every device connected. Most reliable but most expensive. |

🧠 Memory Trick: BSRM = Bus (1 cable), Star (center hub), Ring (circle), Mesh (all connected). Remember: 'Be Smart, Ring Mesh!'

🇧🇩 Bangla Summary: নেটওয়ার্ক টপোলজি হলো নেটওয়ার্কের গঠন বা বিন্যাস। চারটি প্রধান টপোলজি: Bus (একটি তার), Star (কেন্দ্রে হাব), Ring (বৃত্তাকার), Mesh (সব ডিভাইস একে অপরের সাথে সংযুক্ত)।

# Topic 5: Network Components (PINCONA)

A network needs several parts to work. Remember the word: P-I-N-C-O-N-A

⭐ Important: PINCONA = Physical media, Interconnecting devices, Networks software, Computers, Operating system, Networking Applications

## 1. Physical Media (Networking Media)

The medium through which data is sent from one device to another.

- Wired: Twisted pair cables (LAN), Coaxial cable (Cable TV), Fiber optic (long distance, fast)

- Wireless: Wi-Fi, Bluetooth, Satellite, 4G/5G

Real-life example: The cable that connects your laptop to the router is the physical medium.

## 2. Interconnecting Devices

Devices that connect computers and networks together:

- Hub: Sends data to all devices on the network (old technology).

- Switch: Smarter than a hub – sends data only to the correct device.

- Router: Connects different networks together. Sends data between LANs and WANs.

- Wireless Access Point: Provides Wi-Fi connection.

- Modem: Converts digital signals to analog (for telephone lines) and back.

Real-life example: The Wi-Fi router in your home is an interconnecting device.

## 3. Computers: Clients and Servers

In a network, computers play two roles:

- Server: A dedicated computer that provides services (files, printing, email, security) to others. It runs 24/7.

- Client: A computer that requests services from the server. Your laptop is a client.

Real-life example: YouTube's computers are servers. Your phone watching YouTube is the client.

## 4. Networking Software

Software is needed to control how data is sent and received. This includes protocols (rules for communication) and operating system features.

## 5. Applications

These are programs that use the network:

- Web browser (Chrome, Firefox)

- Email client (Gmail)

- Video conferencing apps (Zoom, Teams)

- FTP client (for transferring files)

🧠 Memory Trick: Think of a restaurant: Server = Waiter, Client = Customer, Physical media = Road to deliver food, Router = Traffic police directing cars!

🇧🇩 Bangla Summary: নেটওয়ার্কের উপাদান হলো: Physical Media (তার/বেতার), Interconnecting Devices (HUB, Switch, Router), Computers (Client ও Server), Networking Software, এবং Applications। সার্ভার সেবা দেয়, ক্লায়েন্ট সেবা নেয়।

# Topic 6: Protocol Hierarchies – Layers, Protocols & Interfaces

## What is a Protocol?

A protocol is an agreement or a set of rules between two communicating parties about HOW they will communicate.

⭐ Important: Protocol = Rules of communication. Just like humans follow rules when talking (say 'Hello' first, wait for the other person to reply), computers follow protocols.

## What is a Layer?

Networks are organized in layers. Each layer has a specific job. Higher layers use the services of lower layers.

- Think of layers like a company: The CEO (top layer) gives orders to the Manager (middle layer), who gives orders to Workers (bottom layer).

- Each layer communicates with the layer above and below it.

- The same layer on two different computers can communicate through their protocol.

## Key Terms

Peers

Peers are entities in the same layer on different machines.

Example: Layer 3 on Computer A communicates logically with Layer 3 on Computer B. They are peers.

Interface

An interface is the boundary between two adjacent layers. It defines what services the lower layer provides to the upper layer.

Example: Layer 2 (Data Link) provides an interface to Layer 3 (Network). Layer 3 doesn't need to know HOW Layer 2 works internally.

Entity

An entity is an active element in a layer. It can be software (a process/program) or hardware (a chip), or both.

Peer Entities

Entities in the same layer on different machines are called peer entities. They use the same protocol to communicate.

## Protocol Stack

A protocol stack is a list of protocols used by a system — one protocol for each layer. All the layers together form the protocol stack.

Example: When you browse a website, your computer uses: HTTP (Application) → TCP (Transport) → IP (Network) → Ethernet (Data Link) → Physical cable. This is a protocol stack!

## Connections & Multiplexing

Connection-Oriented Service

- A connection is first established, then used, then released.

- Like a phone call: dial → talk → hang up.

- Example: TCP (Transmission Control Protocol)

Connectionless Service

- No connection is set up beforehand. Data is just sent.

- Like sending a letter by post: just write and send.

- Example: UDP (User Datagram Protocol)

## Multiplexing

Multiplexing means combining multiple connections into one.

- Upward Multiplexing: Several layer N connections are multiplexed (combined) into one layer N-1 connection.

- Downward Multiplexing: One layer N connection uses several layer N-1 connections.

Real-life example: Many people talking on one telephone cable at the same time = multiplexing!

## Service Primitives

Service primitives are the commands used by layers to communicate with each other. There are 4 types:

| Primitive | Meaning / Direction |
| --- | --- |
| REQUEST | Layer N+1 asks Layer N to do something (upper → lower) |
| INDICATION | Layer N tells Layer N+1 that an event happened (lower → upper) |
| RESPONSE | Layer N+1 responds to the previous INDICATION (upper → lower) |
| CONFIRM | Layer N confirms the REQUEST was completed (lower → upper) |

🧠 Memory Trick: Primitives: R-I-R-C = Request, Indication, Response, Confirm. Like a phone call: I call (Request) → You hear it ring (Indication) → You answer (Response) → Connected (Confirm)!

🇧🇩 Bangla Summary: Protocol হলো যোগাযোগের নিয়ম। নেটওয়ার্ককে স্তরে স্তরে (Layer) ভাগ করা হয়। প্রতিটি স্তর একটি নির্দিষ্ট কাজ করে। একই স্তরের entities-রা Peers। স্তরের মধ্যে সীমানাকে Interface বলে। Service Primitives হলো স্তরগুলোর মধ্যে যোগাযোগের কমান্ড।

# Topic 7: The OSI Reference Model

## What is the OSI Model?

OSI stands for Open Systems Interconnection. It was developed by ISO (International Standards Organization).

The OSI model has 7 layers. It is a standard guide for how networks should communicate.

⭐ Important: OSI Model has 7 layers. Remember from top to bottom: Application, Presentation, Session, Transport, Network, Data Link, Physical.

🧠 Memory Trick: Memory trick (top to bottom): 'All People Seem To Need Data Processing' → Application, Presentation, Session, Transport, Network, Data Link, Physical

## Layer 7 – Application Layer

This is the top layer. It is the layer closest to the user.

- Makes sure the other party is identified and can be reached.

- Authenticates the sender or receiver.

- Checks that necessary resources exist (like a modem or internet connection).

- Ensures agreement about error recovery, data integrity, and privacy.

- Sets the protocol and data syntax rules at the application level.

Real-life example: When you open Gmail in Chrome, the browser uses Application Layer protocols (HTTP, SMTP) to communicate.

## Layer 6 – Presentation Layer

This layer handles how data is represented (formatted).

- Translates different data formats between computers.

- Example: Converts ASCII text to the network standard format.

- Handles data encryption and compression.

Real-life example: When you send a photo, it may be compressed (made smaller) by the Presentation Layer so it travels faster.

Think of it as a translator: It makes sure both computers understand each other's data format.

## Layer 5 – Session Layer

This layer manages sessions (conversations) between computers.

- Sets up, coordinates, and ends conversations between applications.

- Controls the exchange of information.

- Provides synchronization (makes sure both sides are in sync).

Real-life example: When you log into a website, a session is created. When you log out, the session is terminated.

## Layer 4 – Transport Layer

This layer ensures reliable data delivery from end to end.

- Accepts data from the Session Layer and splits it into smaller units (segments).

- Passes the segments to the Network Layer.

- Makes sure ALL pieces arrive correctly at the destination.

- Checks the sequence of packets (orders them correctly).

Real-life example: Like a courier service that tracks every parcel and makes sure all parts of a large shipment arrive and are in the right order.

Protocols: TCP (reliable), UDP (fast but unreliable)

## Layer 3 – Network Layer

This layer handles routing – finding the best path for data from source to destination.

- Performs network routing (finding the path).

- Handles flow control and error control.

- Prevents network congestion (too much data clogging the network).

Real-life example: Like a GPS system for data. It finds the best route for your data from Dhaka to London.

Protocol: IP (Internet Protocol)

## Layer 2 – Data Link Layer

This layer provides reliable transfer of data between directly connected nodes.

- Takes the bit stream from the Network Layer and breaks it into frames.

- Passes frames to the Physical Layer for transmission.

- At the receiving end, detects and possibly corrects errors.

- Passes the correct stream back to the Network Layer.

- Also handles flow control.

Real-life example: Like a delivery man who packages letters (frames), delivers them, and makes sure none are lost on the way.

Protocols: HDLC, Ethernet, PPP

## Layer 1 – Physical Layer

This is the bottom layer. It deals with the actual bits being transmitted.

- Transmits raw bits over a communication channel (cable, fiber, wireless).

- Establishes and terminates a physical connection.

- Ensures that when one side sends '1', the other side receives '1' (not '0').

Real-life example: The actual cable, Wi-Fi signal, or fiber optic light pulses that carry the data.

| Layer নং | Layer নাম | প্রধান কাজ | Protocol/Device | Data Unit |
| --- | --- | --- | --- | --- |
| 7 | Application | User Interface, Network Services | HTTP, FTP, SMTP, DNS | Data/Message |
| 6 | Presentation | Translation, Encryption, Compression | JPEG, SSL, TLS | Data |
| 5 | Session | Session Management | NetBIOS, PPTP | Data |
| 4 | Transport | End-to-End Delivery, Flow/Error Control | TCP, UDP | Segment |
| 3 | Network | Routing, Logical Addressing | IP, ICMP, Router | Packet |
| 2 | Data Link | Framing, MAC Addressing, Error Detection | Ethernet, Switch | Frame |
| 1 | Physical | Bit Transmission, Physical Medium | Hub, Cable, NIC | Bit |

| Layer | Name |
| --- | --- |
| 7 | Application Layer – User interaction, HTTP, SMTP, FTP |
| 6 | Presentation Layer – Data format, encryption, compression |
| 5 | Session Layer – Manage sessions, synchronization |
| 4 | Transport Layer – Reliable delivery, TCP/UDP, sequencing |
| 3 | Network Layer – Routing, IP addressing, congestion control |
| 2 | Data Link Layer – Frames, error detection, flow control |
| 1 | Physical Layer – Bits, cables, signals, hardware |

🧠 Memory Trick: Bottom-up memory trick: 'Please Do Not Throw Sausage Pizza Away' → Physical, Data Link, Network, Transport, Session, Presentation, Application

🇧🇩 Bangla Summary: OSI Model এ ৭টি স্তর আছে। নিচ থেকে উপরে: Physical (তার/সংকেত), Data Link (ফ্রেম তৈরি), Network (রাউটিং), Transport (নির্ভরযোগ্য ডেলিভারি), Session (সেশন পরিচালনা), Presentation (ফরম্যাট ও এনক্রিপশন), Application (ব্যবহারকারীর সাথে ইন্টারফেস)।

# Topic 8: The TCP/IP Reference Model

## What is TCP/IP?

TCP/IP stands for Transmission Control Protocol / Internet Protocol. It is the actual model used by the Internet.

Unlike OSI (which has 7 layers), TCP/IP has 4 layers:

| TCP/IP Layer | Equivalent OSI Layers |
| --- | --- |
| Application | Application + Presentation + Session (OSI 7, 6, 5) |
| Transport | Transport (OSI 4) |
| Internet | Network (OSI 3) |
| Network Access / Link | Data Link + Physical (OSI 2, 1) |

## Key Difference: OSI vs TCP/IP

- OSI: 7 layers – theoretical model, good for learning.

- TCP/IP: 4 layers – practical model, actually used on the Internet.

- OSI separates Presentation and Session layers. TCP/IP combines them into Application.

🧠 Memory Trick: TCP/IP = 4 layers: Application, Transport, Internet, Network Access. Remember: A-T-I-N = 'A Tiny Internet Network'

🇧🇩 Bangla Summary: TCP/IP হলো ইন্টারনেটে ব্যবহৃত আসল মডেল। এতে ৪টি স্তর আছে: Application, Transport, Internet, এবং Network Access। OSI মডেল শেখার জন্য, আর TCP/IP বাস্তবে ব্যবহারের জন্য।

# Topic 9: Data Link Control – HDLC

## What is HDLC?

HDLC stands for High-Level Data Link Control. It is a protocol used at the Data Link Layer.

HDLC defines HOW data is packaged into frames and sent reliably between two nodes.

## HDLC Frame Structure

Every HDLC frame has this structure:

| Frame Part | Description |
| --- | --- |
| Flag (01111110) | Marks the beginning and end of every frame. It is a special bit pattern. |
| Address Field | Identifies which terminal the frame is for (on lines with multiple terminals). |
| Control Field | Contains sequence numbers, acknowledgements, and control info. |
| Data Field | The actual information being sent. Can be any length. |
| CRC (Checksum) | Error checking field using Cyclic Redundancy Code technique. |
| End Flag (01111110) | Same as the start flag – marks end of frame. |

⭐ Important: Flag = 01111110. If this bit pattern appears inside the data, byte stuffing is used to avoid confusion. A '0' bit is inserted after every 5 consecutive '1' bits.

## Three Types of HDLC Frames

1. Information Frame (I-Frame)

Used to send actual data.

- SEQ (3 bits): Sequence number of the frame being sent.

- NEXT: Piggybacked acknowledgement – contains the number of the next expected frame.

- P/F bit: Poll/Final bit. P = Polling (asking terminals to send). F = Final frame from terminal.

Uses sliding window protocol. Up to 7 unacknowledged frames can be in transit at the same time (3-bit sequence number → 0 to 7).

2. Supervisory Frame (S-Frame)

Used for control purposes – no user data. There are 4 types:

- Type 00 – RECEIVE READY (RR): I'm ready. The next frame I expect is NEXT.

- Type 01 – REJECT (REJ): Error! Resend all frames starting from NEXT.

- Type 10 – RECEIVE NOT READY (RNR): Stop sending! I am busy. (Also acknowledges frames up to NEXT-1).

- Type 11 – SELECTIVE REJECT (SREJ): Error in one specific frame. Resend only that frame.

3. Unnumbered Frame (U-Frame)

Used for control and for connectionless data transfer.

- DISC (Disconnect): Tell the other side to close the connection.

- SABM (Set Asynchronous Balanced Mode): Reset the line.

- FRMR (Frame Reject): Received a frame with correct checksum but wrong meaning (semantic error).

| Frame Type | Purpose |
| --- | --- |
| Information (I-Frame) | Send actual data with sequence numbers and acknowledgements. |
| Supervisory (S-Frame) | Control: Ready, Reject, Not Ready, Selective Reject. |
| Unnumbered (U-Frame) | Control: Connect, Disconnect, Reset, Error reporting. |

🧠 Memory Trick: HDLC Frame = Flag ¦ Address ¦ Control ¦ Data ¦ CRC ¦ Flag. Remember: 'Friendly ACDs Cheerfully Farewell!' (Flag, Address, Control, Data, CRC, Flag)

🇧🇩 Bangla Summary: HDLC হলো Data Link Layer এর একটি প্রোটোকল। এটি ডেটাকে ফ্রেমে ভাগ করে পাঠায়। প্রতিটি ফ্রেমে থাকে: Flag, Address, Control, Data, CRC, এবং Flag। তিন ধরনের ফ্রেম: Information (ডেটা পাঠানো), Supervisory (নিয়ন্ত্রণ), Unnumbered (সংযোগ ব্যবস্থাপনা)।

📝 EXAM REVISION NOTES

Quick key points for exam preparation:

## Computer Network Basics

- Network = 2+ autonomous computers connected to share resources.

- LAN = Local (room/building). MAN = City. WAN = Country/World.

- Internet = Biggest WAN.

## Network Topology (BSRM)

- Bus: One cable, all devices connected to it. Single point of failure.

- Star: Central hub/switch. Most common. Hub failure = all fail.

- Ring: Circle. Single ring (one way) or Dual ring (two ways).

- Mesh: All-to-all. Most reliable. Most expensive. Used in WANs/banks.

## Protocol, Layers & Interfaces

- Protocol = rules for communication between two parties.

- Peer = same layer on different machines.

- Interface = boundary between adjacent layers.

- Protocol Stack = one protocol per layer, all layers together.

- Service Primitives: Request → Indication → Response → Confirm.

## OSI Model – 7 Layers (top to bottom)

- 7. Application – User interface (HTTP, SMTP, FTP).

- 6. Presentation – Format, encryption, compression.

- 5. Session – Manage sessions.

- 4. Transport – Reliable delivery, TCP, UDP.

- 3. Network – Routing, IP.

- 2. Data Link – Frames, error detection (HDLC, Ethernet).

- 1. Physical – Bits, cables, signals.

## TCP/IP Model – 4 Layers

- Application (= OSI 7+6+5).

- Transport (= OSI 4).

- Internet (= OSI 3).

- Network Access (= OSI 2+1).

## HDLC

- Frame = Flag | Address | Control | Data | CRC | Flag.

- Flag = 01111110.

- 3 frame types: I-Frame (data), S-Frame (control), U-Frame (unnumbered/control).

- S-Frame types: RR (ready), REJ (reject/resend all), RNR (not ready), SREJ (selective reject).

- Sliding window: 3-bit SEQ → up to 7 unacknowledged frames in transit.

⭐ Important: Memory Tricks: • Topologies: B-S-R-M = 'Be Smart, Ring Mesh' • OSI (top→bottom): 'All People Seem To Need Data Processing' • OSI (bottom→up): 'Please Do Not Throw Sausage Pizza Away' • TCP/IP: A-T-I-N = 'A Tiny Internet Network' • HDLC Frame: 'Friendly ACDs Cheerfully Farewell' = Flag, Address, Control, Data, CRC, Flag

— End of Week 1 Notes —

Shaon Khan

IIT-14 JU-52

01633040670