Networking Concepts

Routing, VLAN, and TCP/UDP — Easy Study Notes

Reorganized from Lecture 10 for simple and complete reading

# What This Note Covers

- 1. Routing — what a router does and how it picks a path

- 2. Routing Protocols — static vs dynamic, and common protocol names

- 3. Internetworking Devices — hub, bridge, switch, router

- 4. VLAN — virtual networks and their benefits

- 5. TCP/UDP — the two main transport protocols

Note: Each topic is short and simple on purpose. Read it once, and you will cover everything in the original slides.

# 1. Routing

## 1.1 What Is a Router?

A router is a device. Its job is to decide the next point where a data packet should go, so the packet can reach its final destination.

- A router lets different networks talk to each other.

- A router keeps a table of all the routes it knows, along with their condition. It uses this table to pick the best route for each packet.

- A packet usually passes through several routers before it reaches its destination.

- If there are many possible routes, the router picks the one with the lowest weight (also called metric) first.

Note: Think of a router like a traffic signal for data. It always looks for the shortest and safest path.

## 1.2 How Routing Looks in a Network

Figure 1: A simple picture of how routing connects different networks together.

## 1.3 Routing Protocols

A routing protocol is a set of rules that routers use to share route information with each other. There are two main types.

- Static Routing — the route is set by hand. It does not change on its own.

- Dynamic Routing — routers learn routes automatically and update them when the network changes.

Dynamic routing protocols are further grouped by where they work:

- IGP (Interior Gateway Protocol) — sends route data inside one autonomous system (one organization's network). Examples:

  - RIP (Routing Information Protocol)

  - RIP-2 (RIP Version 2)

  - OSPF (Open Shortest Path First)

  - IGRP (Interior Gateway Routing Protocol)

  - EIGRP (Enhanced Interior Gateway Routing Protocol)

  - IS-IS

- EGP (Exterior Gateway Protocol) — sends route data between different autonomous systems. Example:

  - BGP (Border Gateway Protocol)

Note: An autonomous system just means one network that is managed by one organization, like a company or an internet service provider.

# 2. Internetworking Devices

These are the main devices used to build and connect networks. Each one works at a different level and has a different job.

| Device | What It Does |
| --- | --- |
| Hub | Connects many devices to one network. It repeats and resends the signal to all ports, without checking where it should go. |
| Bridge | Splits one network into smaller parts. It works at Layer 2 (data link layer) and does not depend on higher-level protocols. |
| Switch | Works like a bridge but has more ports. Each port is its own separate segment, so it reduces data collisions and improves speed. |
| Router | Connects different networks and separates broadcast traffic. It sends data based on the network address (Layer 3), not the device's hardware address. |

Note: Simple way to remember: Hub = repeats to everyone. Bridge/Switch = splits traffic smartly. Router = connects separate networks.

# 3. VLAN (Virtual LAN)

## 3.1 What Is a VLAN?

A VLAN lets network managers group users by logic, such as their job, project, or the application they use, instead of grouping them by physical location.

- Data can only move between VLANs through routing.

- VLANs do the same job that physical routers used to do — they divide a LAN into smaller, separate parts.

## 3.2 VLAN and Inter-VLAN Routing

Figure 2: VLANs are separate groups. A router is needed for them to communicate with each other.

## 3.3 Why Use VLANs?

- Broadcast Control — Just like switches keep collision traffic separate, VLANs keep broadcast and multicast traffic inside their own group.

- Security — If routing is turned off for a VLAN, users outside that VLAN cannot reach users inside it, and the other way around. This gives strong protection for sensitive projects.

- Performance — Users who need faster or separate networking can be placed in their own VLAN.

Note: VLANs are useful because they give the benefits of separate physical networks, without needing separate physical cables.

# 4. TCP / UDP

TCP and UDP are both Transport Layer protocols. This layer is responsible for moving data between two devices.

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection Type | Connection-oriented (a link is set up before sending data) | Connectionless (data is sent with no set-up) |
| Reliability | Uses checksum and acknowledgment to check and confirm delivery | No delivery check; faster but less reliable |
| Port Number | Uses a 16-bit source and destination port number | Uses a 16-bit source and destination port number |

## 4.1 Standard Port Numbers

Common applications always use the same port number, so devices know which service to connect to.

- Email — Port 25

- Telnet — Port 23

- FTP — Ports 20 and 21

- SSH — Port 22

Note: TCP is used when accuracy matters, like sending a file. UDP is used when speed matters more than perfect accuracy, like video calls.

# Quick Summary

- Routing moves packets across networks using the best path.

- Routing protocols can be static (fixed) or dynamic (self-updating), and are grouped as IGP or EGP.

- Hubs, bridges, switches, and routers each connect networks in a different way.

- VLANs group users logically for better control, security, and performance.

- TCP is reliable and connection-based. UDP is fast and connectionless.

Note: Reading this summary after the full notes is a good way to check what you remember.