Wireless LAN & Network Devices

Easy notes — explained in very simple words

# Part 1: Wireless LAN (802.11)

## 1. What is a Wireless LAN?

Imagine your computer talking to the internet without any wire. No cable, no string — just invisible radio waves flying through the air. That is a Wireless LAN. It lets devices connect to a network using radio signals instead of cables.

## 2. What is an Access Point (AP)?

An Access Point is like a friendly helper box. It catches the wireless signal from your laptop or desktop, and it also has a wire going to the big network. So it joins two worlds together — the wireless world and the wired world. We can say the AP works like a bridge between wireless devices and the wired network.

A Desktop and a Laptop both talk to the Access Point wirelessly. The Access Point is wired to the bigger network.

- A computer needs a special wireless card (called an 802.11 LAN card) to talk to the Access Point.

- The Access Point has small antennae (like little ears) to send and catch the radio signal.

## 3. How Far Can the Signal Reach? (Range)

The distance between an Access Point and a device is called the range. Walls, doors, and big objects can block the signal, so the range becomes shorter. To cover a big area like a whole building, many Access Points are placed with a little overlap (about 20-30%), like umbrellas overlapping so there is no gap with no signal.

## 4. Hand-Off — Moving Between Access Points

A device always talks to one Access Point at a time. When you walk closer to a different Access Point, your device quietly switches to the new, stronger one. This smooth switch is called Hand-Off. It is just like changing hands while carrying a baby — the baby never falls, it is passed gently from one arm to another.

## 5. Three Flavors of Wireless LAN

Wireless LAN comes in three common types, called 802.11a, 802.11b and 802.11g. Think of them as three different flavors of the same ice cream — same idea, but each one a little different in speed and the road (frequency band) they travel on.

## 5.1 802.11b

- The most common and most used type.

- Speed: 1, 2, 5.5 or 11 Mbps (megabits per second).

- Travels on the 2.4 GHz road, called the ISM band (Industrial-Scientific-Medical).

## 5.2 802.11a

- Travels on a different road: the 5 GHz band, called UNII.

- It cannot talk to 2.4 GHz devices — different road, no meeting.

- Faster: up to 54 Mbps.

## 5.3 802.11g

- Also fast: up to 54 Mbps.

- But it travels on the 2.4 GHz road, same as 802.11b.

- Because it shares the same road, it can also talk nicely with older 802.11b devices (backward compatible).

## 6. Multiple Access with Collision Avoidance (MACA)

When many devices want to talk over the air at the same time, their words can crash into each other — like two people shouting at once. MACA is a polite conversation rule that stops this crashing (collision).

Sender and receiver politely take turns using RTS, CTS, data and ACK signals.

Here is the simple, step-by-step polite conversation, before any data is sent:

- Step 1: The sender first sends a small note called RTS (Request to Send). It says 'Hey, can I talk, and for how long?'

- Step 2: The receiver replies with CTS (Clear to Send). It says 'Yes, go ahead, I am listening.'

- Step 3: Now the sender sends the real data.

- Step 4: The receiver sends back an ACK (Acknowledgement), meaning 'I got it!' Now someone else may talk.

- If the sender does not get a CTS reply back, it thinks: 'Oops, maybe my note crashed with someone else.' This is called a collision.

# Part 2: Repeater, Hub, Bridge and Switch

These four devices help carry data along a network. Think of them like helpers passing a parcel from one house to another. Each helper has a different, smarter way of passing it along.

## 1. Repeater

A repeater is like a person with a megaphone. The signal gets weak and quiet after traveling far, so the repeater listens to it, makes it strong again, and shouts it onward. It does not understand the message; it just makes it loud and clear again so it can travel further.

- It works at the lowest, simplest level (the Physical Layer).

- Rule of Four: For 10 Mbps Ethernet, no more than four repeaters should be used in a row between two computers.

- Why this rule? Each repeater adds a tiny delay. Too many repeaters in a row makes the message take too long to arrive.

## 2. Hub

A hub is like a megaphone with many speakers attached. It joins many computers together, and it is really just a repeater with multiple ports. The trick is: when a hub gets a message from one computer, it shouts that same message out to every other computer connected to it — except the one that sent it.

- It changes the shape of the network from a straight line (bus) into a star shape, with the hub in the middle.

- Everyone hears everything — it is not very private or efficient.

## 3. Bridge

A bridge is smarter than a hub. It does not just shout everything to everyone. It looks at the address label on each message (called a MAC address) and decides: should I let this message pass to the other side, or should I keep it here? This way, it keeps two parts of a network separate but connected when needed.

- It works one level smarter than a repeater (the Data Link Layer).

- It checks a table of addresses to decide: block it, copy it onward, or send it everywhere (flood).

- Messages meant for everyone (broadcast messages) are always passed along.

## 4. Switch

A switch is like a bridge with many doors (ports), one door for each computer. It is basically a multi-port bridge. Each computer gets its very own private road to the switch, so messages do not crash into each other.

Each computer connects to the switch directly. A talks to A', B talks to B', and C talks to C' — all at the same time, with no crashing.

- Each computer has its own direct wire to the switch (dedicated access).

- Full Duplex means talking and listening can happen at the same time, with no collisions.

- Many pairs of computers (A-A', B-B', C-C') can talk at the very same moment without bumping into each other.

- Switches learn which computer is on which port, building a table, just like a bridge, but for many ports.

- You can connect switches together (cascade them) to make the network even bigger.

# Quick Summary

- Wireless LAN lets devices connect using radio waves; the Access Point joins wireless and wired networks.

- 802.11b, 802.11a and 802.11g are three flavors with different speeds and radio roads.

- MACA uses RTS, CTS, data and ACK messages so wireless devices take turns and avoid crashing into each other.

- Repeater: makes a weak signal strong again.

- Hub: a repeater with many ports; shouts to everyone.

- Bridge: looks at addresses and smartly decides where a message should go.

- Switch: a smart bridge with many ports; gives each computer its own private, collision-free path.