68. Differentiate between ATM cell and TCP packet.

Answer

ATM cells and TCP packets operate at different layers and handle data formatting differently:

| Feature | ATM Cell PDF | TCP Packet / Segment PDF |
| --- | --- | --- |
| OSI Layer | Data Link Layer (Layer 2) | Transport Layer (Layer 4) |
| Size Format | Fixed-size: Exactly 53 bytes | Variable-size: Can vary based on payload and MTU |
| Header Size | 5 bytes | 20 to 60 bytes |
| Data Switching | Switched using virtual circuits (VPI/VCI) | Routed using IP addresses and Port numbers |
| Primary Focus | Fast hardware switching in high-speed links | End-to-end reliable data delivery between processes |

69. Describe both the ATM cell types with appropriate cell formation figure.

Answer

ATM defines two cell formats depending on where the connection is located:

- UNI (User-Network Interface): Used between a user device (host) and an ATM switch.

- NNI (Network-Network Interface): Used between two ATM switches inside the backbone network.

ATM Cell Structure (UNI Format):

An ATM cell consists of a 5-byte header and a 48-byte payload.

Field Description:

- GFC (4 bits): Generic Flow Control (present only in UNI, replaced by extra VPI bits in NNI).

- VPI (8 bits in UNI / 12 bits in NNI): Virtual Path Identifier.

- VCI (16 bits): Virtual Channel Identifier (VPI & VCI guide cells through switches).

- PT (3 bits): Payload Type (distinguishes user data from management cells).

- CLP (1 bit): Cell Loss Priority (if 1, cell can be dropped during congestion).

- HEC (8 bits): Header Error Control (CRC checksum for header only).

70. Write short note on ATM network and transport layer.

Answer

- ATM Network Layer: ATM provides a connection-oriented, cell-switched service. It uses Virtual Paths (VPI) and Virtual Channels (VCI) to establish Virtual Circuits (VC) before sending cells. This ensures predictable routing and Quality of Service (QoS) guarantees for voice, video, and data.

- ATM Transport Layer (AAL - ATM Adaptation Layer): Since higher-layer protocols send large variable-size packets (e.g., IP packets), the ATM Adaptation Layer acts as an interface between upper layers and the ATM layer. It segments variable-length packets into 48-byte cell payloads at the sender side and reassembles them back into packets at the receiver side.

71. How does TCP/UDP (transport layer protocol) works?

Answer

Transport layer protocols manage communication between processes running on different hosts using Port Numbers:

- TCP (Transmission Control Protocol):

  - Works in 3 steps: Connection Setup (3-way handshake) → Data Transfer → Connection Termination.

  - Ensures data arrives in order without errors using sequence numbers, acknowledgments (ACK), and retransmissions.

- UDP (User Datagram Protocol):

  - Operates on a fire-and-forget (connectionless) principle.

  - Takes data from the application layer, attaches port numbers and a simple checksum, and immediately sends it out without setting up a connection or waiting for ACKs.

72. Why would anyone use UDP? Clarify your answer with some applications where the UDP is used.

Answer

UDP is used because it provides low latency, zero connection overhead, and fast speed. Unlike TCP, UDP does not spend time establishing connections, tracking sequence numbers, or waiting for acknowledgments.

Why UDP is preferred in certain scenarios:

- Speed over Reliability: Missing a single packet is better than delaying real-time data.

- Low Overhead: Simple 8-byte header compared to TCP's 20-byte header.

- Supports Broadcast & Multicast: Can send data to multiple receivers at once.

Applications using UDP:

- Real-time Voice/Video Streaming: Zoom, WhatsApp voice calls, YouTube live streams.

- Online Gaming: Fast player movements require low latency.

- DNS (Domain Name System): Requires small, single request-reply queries.

- DHCP: Used for quick IP assignment on bootup.

73. What is the difference between UDP and TCP internet protocols?

Answer

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection Type | Connection-Oriented (Handshake required) | Connectionless (No handshake) |
| Reliability | Guaranteed delivery (Uses ACKs and retransmission) | Best-effort delivery (Packets can be lost) |
| Packet Order | Delivers packets in exact order | Packets may arrive out of order |
| Header Size | 20 to 60 bytes | 8 bytes |
| Flow/Congestion Control | Supported (Sliding Window, Slow Start) | Not supported |
| Speed | Slower (due to overhead) | Very Fast |

74. What are the Challenges of Reliable Data Transfer?

Answer

Achieving reliable data transfer over an unreliable physical network poses several challenges:

- Packet Corruption: Bits can be flipped or corrupted by noise in physical transmission media.

- Packet Loss: Router queues can overflow during network congestion, dropping packets.

- Out-of-Order Delivery: Packets taking different network paths may arrive out of sequence.

- Duplicate Packets: Delayed acknowledgments can cause senders to retransmit packets that were already received.

- Flow Mismatch: A fast sender can overwhelm a slow receiver's buffer space.

75. How TCP supports for TCP Support for Reliable Delivery?

Answer

TCP provides reliable data delivery over unreliable IP networks using five core mechanisms:

- Checksum: Detects bit errors/corruption in headers and data.

- Sequence Numbers: Keeps track of bytes sent so the receiver can reorder packets and detect duplicates.

- Acknowledgments (ACK): Receiver sends ACK numbers back to inform the sender which bytes were received successfully.

- Timeouts & Retransmissions: If an ACK is not received within a calculated RTT (Round Trip Time) window, TCP automatically retransmits the lost packet.

- Flow Control (Sliding Window): Prevents the sender from overflowing the receiver's buffer by adjusting the window size based on receiver feedback.

Q112: Differentiate between 1G, 2G, 3G and 4G.

Answer

Mobile telephone systems evolved through four major generations:

| Feature | 1G PDF | 2G PDF | 3G PDF | 4G PDF |
| --- | --- | --- | --- | --- |
| Era | 1980s | 1990s | 2000s | 2010s |
| Technology | Analog | Digital | Digital | Digital (All-IP) |
| Services | Voice only | Voice + SMS | Voice + Mobile Internet | Broadband Internet + VoLTE |
| Modulation | FM | QPSK / TDMA | CDMA (W-CDMA) | OFDM |
| Data Speed | ~2.4 kbps | ~384 kbps | ~2 Mbps | 100+ Mbps |
| Example | AMPS | GSM | UMTS | LTE |

Q113: Write short notes on IMPS and AMPS.

Answer

- IMPS (Improved Mobile Telephone Service): An early pre-cellular mobile telephone system introduced in the 1960s. It used a single high-power central transmitter covering an entire city with a limited number of channels, allowing very few simultaneous calls.

- AMPS (Advanced Mobile Phone System): The 1G analog cellular network standard introduced in the 1980s. It divided coverage areas into small hexagonal cells, allowing frequency reuse across different non-adjacent cells. It operated in the 800 MHz band with 832 full-duplex channels using Frequency Modulation (FM).

Q114: How AMPS accumulate more calls than IMPS?

Answer

AMPS accommodated significantly more calls than IMPS through two main mechanisms:

- Cellular Frequency Reuse: Instead of using one big central tower covering an entire city (like IMPS), AMPS divided geographical areas into many small hexagonal cells. Non-adjacent cells could reuse the same frequency channels simultaneously without causing signal interference.

- Cell Splitting: As the number of subscribers grew in a particular area, large cells were subdivided into smaller microcells. Each microcell reused frequencies independently, multiplying overall network capacity.

Q107. How to Measure the Network Performance?

Network performance means how well a network works. It is measured using the following factors:

- Bandwidth: The maximum amount of data the network can carry.

- Throughput: The actual amount of data transferred in a given time.

- Latency (Delay): The time taken for data to travel from sender to receiver.

- Jitter: The variation in packet delay.

- Packet Loss: The number of packets lost during transmission.

- Reliability: How stable and dependable the network is.

A network with high throughput, low delay, low jitter, and low packet loss has good performance.

Q142. Write Short Note on SNMP, MIBs, TRAP. What is the Purpose of MIBs?

SNMP (Simple Network Management Protocol)

SNMP is a protocol used to monitor and manage network devices such as routers, switches, and servers from a central location.

MIB (Management Information Base)

MIB is a database that stores information about network devices. The SNMP manager reads this information to monitor device status.

Purpose of MIB:

- Stores device information.

- Helps monitor network performance.

- Makes network management easier.

- Allows the manager to read or update device information.

TRAP

A TRAP is a message sent automatically by an SNMP agent to the SNMP manager when an important event or problem occurs.

Example: Device failure or link down.

Q143. Show SNMP Manager and SNMP Agent Communication Using SNMP Protocol. Explain Each Message.

SNMP works by exchanging messages between the Manager and the Agent.

Simple Diagram

SNMP Messages

- Get Request: Manager requests information from the agent.

- Get Response: Agent sends the requested information.

- Set Request: Manager changes or updates a value in the device.

- Trap: Agent automatically sends an alert when an important event happens.

Q111. Explain the Characteristics of Enterprise Network and Design Issues for this Network.

Characteristics of an Enterprise Network

An enterprise network is a network used by an organization or company to connect its employees, offices, and devices.

Characteristics:

- Connects many users and devices.

- High-speed communication.

- Secure and reliable.

- Easy to expand.

- Supports wired and wireless connections.

- Allows resource sharing.

Design Issues of an Enterprise Network

When designing an enterprise network, the following issues should be considered:

- Scalability: The network should support future growth.

- Reliability: The network should continue working even if one device fails.

- Security: Protect data from unauthorized access.

- Performance: Provide high speed with low delay.

- Cost: The network should be cost-effective.

- Easy Management: The network should be easy to monitor and maintain.