Jahangirnagar University

Institute of Information Technology

Savar, Dhaka-1342

Project report

over

XYZ Company-Savar Branch Network Design

Course: Computer Networks Lab

Course Code: ICT-3104

Submitted To

Jesmin Akhter, PhD

Professor, Institute of Information Technology

Submitted By

Md. Shaon Khan

ID: 1984

Session: 2022–2023

Date of Submission

7 May 2026

TABLE OF CONTENTS

Contents

1. INTRODUCTION	4

2. PROJECT OBJECTIVES	5

3. NETWORK REQUIREMENTS	5

4. NETWORK TOPOLOGY	6

5. IP ADDRESSING AND VLAN PLAN	7

6. DEVICE AND PORT CONNECTION PLAN	8

7. PHYSICAL CABLING	8

8. SWITCH CONFIGURATION	8

8.1 Enter Configuration Mode	8

8.2 Create VLANs	9

8.3 Configure Router Link as Trunk	9

8.4 Configure Admin/IT Access Ports	9

8.5 Configure Finance/HR Access Ports	9

8.6 Configure Customer Service Access Ports	9

8.7 Configure Access Point Ports	10

8.8 Save Configuration	10

9. SWITCH VERIFICATION	10

10. ROUTER-ON-A-STICK CONFIGURATION	11

10.1 Enable Physical Interface	11

10.2 VLAN 10 Subinterface	11

10.3 VLAN 20 Subinterface	11

10.4 VLAN 30 Subinterface	12

10.5 Native VLAN 99 Subinterface	12

11. DHCP CONFIGURATION	12

11.1 Excluded Addresses	12

11.2 Admin/IT DHCP	12

11.3 Finance/HR DHCP	12

11.4 Customer Service DHCP	12

11.5 Save Configuration	13

12. WIRELESS ACCESS POINT CONFIGURATION	13

13. CLIENT CONFIGURATION	14

13.1 Wired Clients	14

13.2 Wireless Clients	14

14. DHCP VERIFICATION	14

15. ROUTER INTERFACE VERIFICATION	15

16. NETWORK TESTING	15

16.1 Router to VLAN Gateways	15

16.2 Router to Wireless Client	15

17. INTER-VLAN ROUTING VERIFICATION	15

18. FINAL VERIFICATION SUMMARY	17

19. SECURITY CONSIDERATIONS	17

20. TROUBLESHOOTING AND RESOLUTION	17

21. CONCLUSION	18

22. REFERENCES	18

APPENDIX A: COMPLETE SWITCH CONFIGURATION	18

APPENDIX B: COMPLETE FINAL ROUTER CONFIGURATION	19

APPENDIX C: VERIFICATION COMMANDS	21

Switch	21

Router	21

Client	21

# 1. INTRODUCTION

This project presents the design and implementation of a computer network for the Savar branch of XYZ Company using Cisco Packet Tracer. The purpose of the project is to build a structured, department-wise network that allows employees of different departments to communicate efficiently while keeping their traffic logically separated for better organization and control.

A company branch with multiple departments requires a structured network rather than a flat network because a flat network mixes broadcast traffic from all departments together, which increases congestion and makes management difficult. A structured network divides the organization into logical groups so that each department has its own broadcast domain and can be managed independently.

Virtual Local Area Networks (VLANs) are used in this project to separate the Admin/IT, Finance/HR and Customer Service departments at Layer 2, even though all departments share the same physical switch. VLANs reduce unnecessary broadcast traffic and provide a basic level of traffic isolation between departments.

Since each VLAN is a separate broadcast domain, devices in different VLANs cannot communicate with each other without a routing function. Inter-VLAN communication is therefore required so that, for example, the Finance/HR department can exchange data with the Customer Service department when necessary. This project uses the Router-on-a-Stick method to achieve inter-VLAN routing using a single router interface with subinterfaces.

Dynamic Host Configuration Protocol (DHCP) is used so that end devices automatically receive an IP address, subnet mask, default gateway and DNS server address without manual configuration on each device. This reduces administrative effort and configuration errors, especially as the number of devices increases.

Wireless connectivity is also included in the design because modern offices require mobility for staff who use laptops. Each department is provided with its own wireless access point and SSID so that wireless clients join the correct VLAN based on the department they belong to.

Cisco Packet Tracer is used as the simulation platform for this project. It allows routers, switches, access points, PCs and laptops to be configured and tested in a virtual environment before actual deployment, which makes it a suitable tool for academic networking projects.

# 2. PROJECT OBJECTIVES

The main objectives of this project are as follows:

1. Design a network for XYZ Company's Savar branch.

2. Divide departments into separate VLANs.

3. Provide wired and wireless connectivity.

4. Configure DHCP for automatic IP assignment.

5. Configure Router-on-a-Stick for inter-VLAN routing.

6. Configure trunk communication between router and switch.

7. Provide wireless access for each department.

8. Verify communication between different departments.

9. Test the final network using Cisco Packet Tracer.

# 3. NETWORK REQUIREMENTS

Table 1 summarizes the requirements of the project and how each requirement was implemented.

| Requirement | Implementation |
| --- | --- |
| Router | One Cisco Router 2911 (R-SAVAR) |
| Layer 2 Switch | One Cisco Switch 2960 (SW-SAVAR) |
| Access Points | Three access points (AP1, AP2, AP3) |
| Wired PCs | Six PCs (PC0–PC5) |
| Wireless Laptops | Three laptops (Laptop0–Laptop2) |
| Department VLANs | Three VLANs: Admin/IT, Finance/HR, Customer Service |
| Management/Native VLAN | One native VLAN (VLAN 99) |
| DHCP | Configured on R-SAVAR for all department VLANs |
| Router-on-a-Stick | Configured on R-SAVAR using subinterfaces |
| Inter-VLAN Communication | Achieved through router subinterfaces |
| Wireless Security | WPA2-PSK on each access point |

# 4. NETWORK TOPOLOGY

Figure 1 shows the logical layout of the network, illustrating how the router, switch, switches connecting each access point, wired PCs and wireless laptops are interconnected.

Figure 1: Logical Network Diagram of the XYZ Company Savar Branch Network

The network topology of the XYZ Company Savar branch, as implemented in Cisco Packet Tracer, is shown in Figure 2. The topology was built and tested in Cisco Packet Tracer.

Figure 2: Complete XYZ Company Savar Branch Network Topology

The topology consists of one router, R-SAVAR (Cisco Router 2911), connected through its Gi0/0 interface to the Gi0/1 interface of the Layer 2 switch, SW-SAVAR (Cisco Switch 2960). The router-to-switch link is configured as a trunk so that it can carry traffic for multiple VLANs on a single physical link.

Three wireless access points are connected to the switch: AP1 serves the Admin/IT department, AP2 serves the Finance/HR department, and AP3 serves the Customer Service department. Each access point is connected to a switch port that belongs to the VLAN of its respective department.

Wired clients PC0 and PC1 belong to the Admin/IT department, PC2 and PC3 belong to the Finance/HR department, and PC4 and PC5 belong to the Customer Service department. Wireless clients Laptop0, Laptop1 and Laptop2 connect through AP1, AP2 and AP3 respectively, placing them in the Admin/IT, Finance/HR and Customer Service VLANs.

# 5. IP ADDRESSING AND VLAN PLAN

The base network assigned to the Savar branch is 192.168.1.0/24. This network is divided into four /26 subnets so that each department, along with the management/native VLAN, receives its own subnet.

The subnet mask used for each subnet is 255.255.255.192, which provides 62 usable host addresses per subnet. This is sufficient for the number of devices in each department while keeping the addressing plan simple.

Table 2 shows the complete IP addressing and VLAN plan used in this project.

| Department | VLAN ID | VLAN Name | Network | Usable Host Range | Gateway |
| --- | --- | --- | --- | --- | --- |
| Admin/IT | 10 | ADMIN_IT | 192.168.1.0/26 | 192.168.1.1–192.168.1.62 | 192.168.1.1 |
| Finance/HR | 20 | FINANCE_HR | 192.168.1.64/26 | 192.168.1.65–192.168.1.126 | 192.168.1.65 |
| Customer Service/Reception | 30 | CUSTSERVICE | 192.168.1.128/26 | 192.168.1.129–192.168.1.190 | 192.168.1.129 |
| Management/Native | 99 | MGMT_NATIVE | 192.168.1.192/26 | 192.168.1.193–192.168.1.254 | 192.168.1.193 |

Separate VLANs are used for each department so that broadcast traffic generated within one department does not affect the other departments. VLANs also allow different security and access policies to be applied to each department if required in the future, and they make the network easier to manage and troubleshoot as the company grows.

# 6. DEVICE AND PORT CONNECTION PLAN

Table 3 lists the switch port assignments for every device in the network.

| Device | Switch Port | VLAN | Purpose |
| --- | --- | --- | --- |
| PC0 | Fa0/2 | 10 | Admin/IT |
| PC1 | Fa0/3 | 10 | Admin/IT |
| PC2 | Fa0/4 | 20 | Finance/HR |
| PC3 | Fa0/5 | 20 | Finance/HR |
| PC4 | Fa0/6 | 30 | Customer Service |
| PC5 | Fa0/7 | 30 | Customer Service |
| AP1 | Fa0/10 | 10 | Admin/IT Wi-Fi |
| AP2 | Fa0/11 | 20 | Finance/HR Wi-Fi |
| AP3 | Fa0/12 | 30 | Customer Service Wi-Fi |
| R-SAVAR | Gi0/0 ↔ SW-SAVAR Gi0/1 | Trunk | Router-on-a-Stick |

The link between the router and the switch is configured as a trunk port because it must carry tagged traffic for all four VLANs. All other ports connecting end devices and access points are configured as access ports, since each of these devices belongs to only one VLAN.

# 7. PHYSICAL CABLING

All connections in this project use Copper Straight-Through cables, which is the correct cable type recommended by Cisco Packet Tracer for the following connections:

-  Router Gi0/0 to Switch Gi0/1 – carries trunk traffic between the router and the switch.

-  PC to Switch – connects each wired PC to its assigned access port.

-  Access Point to Switch – connects each access point to its assigned access port.

The physical layout of these connections is visible in the topology diagram shown earlier in Figure 2.

# 8. SWITCH CONFIGURATION

This section explains the configuration of the switch SW-SAVAR step by step.

## 8.1 Enter Configuration Mode

enable configure terminal hostname SW-SAVAR

## 8.2 Create VLANs

vlan 10 name ADMIN_IT exit vlan 20 name FINANCE_HR exit vlan 30 name CUSTSERVICE exit vlan 99 name MGMT_NATIVE exit

VLAN 10 (ADMIN_IT) is used for the Admin/IT department, VLAN 20 (FINANCE_HR) for the Finance/HR department, VLAN 30 (CUSTSERVICE) for the Customer Service department, and VLAN 99 (MGMT_NATIVE) is used as the native/management VLAN for the trunk link.

## 8.3 Configure Router Link as Trunk

interface gigabitEthernet 0/1 switchport mode trunk switchport trunk native vlan 99 exit

Interface Gi0/1 connects the switch to the router and is configured as a trunk port so that it can carry traffic for multiple VLANs. VLAN 99 is set as the native VLAN for this trunk.

## 8.4 Configure Admin/IT Access Ports

interface range fastEthernet 0/2 - 3 switchport mode access switchport access vlan 10 exit

## 8.5 Configure Finance/HR Access Ports

interface range fastEthernet 0/4 - 5 switchport mode access switchport access vlan 20 exit

## 8.6 Configure Customer Service Access Ports

interface range fastEthernet 0/6 - 7 switchport mode access switchport access vlan 30 exit

## 8.7 Configure Access Point Ports

AP1:

interface fastEthernet 0/10 switchport mode access switchport access vlan 10 exit

AP2:

interface fastEthernet 0/11 switchport mode access switchport access vlan 20 exit

AP3:

interface fastEthernet 0/12 switchport mode access switchport access vlan 30 exit

## 8.8 Save Configuration

end copy running-config startup-config

# 9. SWITCH VERIFICATION

The command show vlan brief was used to verify VLAN creation and port assignment on SW-SAVAR. The actual output is shown in Figure 3.

Figure 3: VLAN configuration and port assignments on SW-SAVAR.

The output confirms that VLAN 10 (ADMIN_IT) includes ports Fa0/2, Fa0/3 and Fa0/10; VLAN 20 (FINANCE_HR) includes ports Fa0/4, Fa0/5 and Fa0/11; VLAN 30 (CUSTSERVICE) includes ports Fa0/6, Fa0/7 and Fa0/12; and VLAN 99 (MGMT_NATIVE) is active on the switch.

The command show interfaces trunk was then used to verify the trunk link between the switch and the router. The actual output is shown in Figure 4.

Figure 4: Trunk configuration of SW-SAVAR.

The output confirms that interface Gi0/1 is operating in 802.1Q trunking mode with a native VLAN of 99, and that VLANs 10, 20, 30 and 99 are allowed and active on the trunk.

# 10. ROUTER-ON-A-STICK CONFIGURATION

Router-on-a-Stick allows a single physical router interface to route traffic between multiple VLANs by creating logical subinterfaces, each associated with a specific VLAN through 802.1Q encapsulation. In this project, the physical interface Gi0/0 of R-SAVAR connects to the trunk port of the switch, and four subinterfaces are created under it.

## 10.1 Enable Physical Interface

enable configure terminal hostname R-SAVAR interface gigabitEthernet 0/0 no shutdown exit

## 10.2 VLAN 10 Subinterface

interface gigabitEthernet 0/0.10 encapsulation dot1Q 10 ip address 192.168.1.1 255.255.255.192 exit

## 10.3 VLAN 20 Subinterface

interface gigabitEthernet 0/0.20 encapsulation dot1Q 20 ip address 192.168.1.65 255.255.255.192 exit

## 10.4 VLAN 30 Subinterface

interface gigabitEthernet 0/0.30 encapsulation dot1Q 30 ip address 192.168.1.129 255.255.255.192 exit

## 10.5 Native VLAN 99 Subinterface

interface gigabitEthernet 0/0.99 encapsulation dot1Q 99 native ip address 192.168.1.193 255.255.255.192 exit

Each subinterface acts as the default gateway for the devices in its corresponding VLAN, allowing traffic from one VLAN to be routed to another through R-SAVAR.

# 11. DHCP CONFIGURATION

The router R-SAVAR is configured to provide automatic IP addresses to client devices in each department VLAN using DHCP pools.

## 11.1 Excluded Addresses

ip dhcp excluded-address 192.168.1.1 192.168.1.10 ip dhcp excluded-address 192.168.1.65 192.168.1.75 ip dhcp excluded-address 192.168.1.129 192.168.1.139

These commands exclude the gateway addresses and a reserved range in each subnet from automatic assignment by DHCP.

## 11.2 Admin/IT DHCP

ip dhcp pool ADMIN_IT_POOL network 192.168.1.0 255.255.255.192 default-router 192.168.1.1 dns-server 8.8.8.8 exit

## 11.3 Finance/HR DHCP

ip dhcp pool FINANCE_HR_POOL network 192.168.1.64 255.255.255.192 default-router 192.168.1.65 dns-server 8.8.8.8 exit

## 11.4 Customer Service DHCP

ip dhcp pool CUSTSERVICE_POOL network 192.168.1.128 255.255.255.192 default-router 192.168.1.129 dns-server 8.8.8.8 exit

## 11.5 Save Configuration

end copy running-config startup-config

# 12. WIRELESS ACCESS POINT CONFIGURATION

Each access point is connected to a switch access port that belongs to the VLAN of its department, as summarized in Table 4.

| Access Point | Switch Port | VLAN | SSID | Security |
| --- | --- | --- | --- | --- |
| AP1 | Fa0/10 | 10 | ADMIN_IT_WIFI | WPA2-PSK |
| AP2 | Fa0/11 | 20 | FINANCE_HR_WIFI | WPA2-PSK |
| AP3 | Fa0/12 | 30 | CUSTSERVICE_WIFI | WPA2-PSK |

AP1 is configured with SSID ADMIN_IT_WIFI and WPA2-PSK security for the Admin/IT department. The configuration screen is shown in Figure 5.

Figure 5: Wireless configuration of AP1 (ADMIN_IT_WIFI).

AP2 is configured with SSID FINANCE_HR_WIFI and WPA2-PSK security for the Finance/HR department. The configuration screen is shown in Figure 6.

Figure 6: Wireless configuration of AP2 (FINANCE_HR_WIFI).

AP3 is configured with SSID CUSTSERVICE_WIFI and WPA2-PSK security for the Customer Service department. The configuration screen is shown in Figure 7.

Figure 7: Wireless configuration of AP3 (CUSTSERVICE_WIFI).

# 13. CLIENT CONFIGURATION

## 13.1 Wired Clients

For each wired PC, the IP configuration was set through PC > Desktop > IP Configuration > DHCP. Once DHCP is selected, the PC automatically receives an IP address, subnet mask, default gateway and DNS server from the router.

## 13.2 Wireless Clients

For each wireless laptop, the following steps were performed: Laptop > Desktop > PC Wireless > Connect, then the appropriate SSID was selected, the WPA2 password was entered, and the IP configuration was set to DHCP.

Laptop0 connects to SSID ADMIN_IT_WIFI and joins VLAN 10. Laptop1 connects to SSID FINANCE_HR_WIFI and joins VLAN 20. Laptop2 connects to SSID CUSTSERVICE_WIFI and joins VLAN 30.

# 14. DHCP VERIFICATION

The command show ip dhcp binding was executed on R-SAVAR to verify that IP addresses were correctly leased to client devices. The actual output is shown in Figure 8.

Figure 8: DHCP bindings assigned by R-SAVAR.

The output confirms that the router successfully assigned IP addresses to nine client devices across the three department VLANs. In VLAN 10 (Admin/IT), the addresses 192.168.1.11, 192.168.1.12 and 192.168.1.13 were leased. In VLAN 20 (Finance/HR), the addresses 192.168.1.76, 192.168.1.77 and 192.168.1.78 were leased. In VLAN 30 (Customer Service), the addresses 192.168.1.140, 192.168.1.141 and 192.168.1.142 were leased. All addresses shown in Figure 8 were automatically assigned by DHCP with an automatic lease type.

# 15. ROUTER INTERFACE VERIFICATION

The command show ip interface brief was executed on R-SAVAR to verify the status of the subinterfaces. The actual output is shown in Figure 9.

Figure 9: Router subinterface status on R-SAVAR.

| Interface | IP Address | Status |
| --- | --- | --- |
| G0/0.10 | 192.168.1.1 | up/up |
| G0/0.20 | 192.168.1.65 | up/up |
| G0/0.30 | 192.168.1.129 | up/up |
| G0/0.99 | 192.168.1.193 | up/up |

The up/up status of every subinterface confirms that the Router-on-a-Stick configuration is operational and ready to route traffic between the department VLANs.

# 16. NETWORK TESTING

The ping command was used from various devices to verify connectivity across the network. The actual results obtained are described below.

## 16.1 Router to VLAN Gateways

Ping tests were carried out to the Finance/HR gateway (192.168.1.65) and the Customer Service gateway (192.168.1.129). Both tests returned a success rate of 100 percent, confirming that the router interfaces for these VLANs were reachable.

## 16.2 Router to Wireless Client

A ping test to the wireless client address 192.168.1.11 also returned a success rate of 100 percent, confirming communication between the router and the Admin/IT wireless client.

# 17. INTER-VLAN ROUTING VERIFICATION

Devices belonging to different VLANs were tested using the ping command to confirm that inter-VLAN routing through R-SAVAR was functioning correctly.

PC0, located in VLAN 10 (Admin/IT), was used to ping 192.168.1.76 (a Finance/HR host in VLAN 20) and 192.168.1.140 (a Customer Service host in VLAN 30). The actual results are shown in Figure 10.

Figure 10: Inter-VLAN connectivity test from PC0 (VLAN 10) to VLAN 20 and VLAN 30 hosts.

Both ping tests in Figure 10 completed with 4 packets sent, 4 received and 0 lost, confirming a 100 percent success rate and successful inter-VLAN routing between VLAN 10, VLAN 20 and VLAN 30.

Laptop1, located in VLAN 20 (Finance/HR), was used to ping 192.168.1.129 (the VLAN 30 gateway) and 192.168.1.1 (the VLAN 10 gateway). The actual results are shown in Figure 11.

Figure 11: Inter-VLAN connectivity test from Laptop1 (VLAN 20) to the VLAN 30 and VLAN 10 gateways.

Both ping tests in Figure 11 also completed with a 100 percent success rate. These results confirm that the router successfully forwards traffic between the Admin/IT, Finance/HR and Customer Service VLANs through the Router-on-a-Stick configuration.

# 18. FINAL VERIFICATION SUMMARY

Table 5 summarizes the verification tests performed during the project, based on the actual results obtained.

| Test | Result |
| --- | --- |
| VLAN creation | Successful |
| Access port assignment | Successful |
| Trunk configuration | Successful |
| Native VLAN 99 | Successful |
| Router subinterfaces | Successful |
| DHCP | Successful |
| Wired client addressing | Successful |
| Wireless connectivity | Successful |
| Inter-VLAN routing | Successful |
| Ping verification | Successful |

# 19. SECURITY CONSIDERATIONS

The following basic security measures were applied in this project:

-  VLAN segmentation separates departments logically, limiting broadcast traffic and unnecessary visibility between departments.

-  WPA2-PSK encryption is used on every wireless access point to secure wireless connections.

-  DHCP provides controlled and consistent automatic addressing across the network.

-  The native VLAN (VLAN 99) is kept separate from the department VLANs on the trunk link.

-  Inter-VLAN communication is centrally handled by the router, allowing traffic between departments to be monitored and controlled at a single point.

No advanced security features such as access control lists, port security or firewall rules were configured in this project, as these were outside the defined scope.

# 20. TROUBLESHOOTING AND RESOLUTION

During implementation, an issue was encountered with the Admin/IT wireless laptop. Although the laptop received an IP address from DHCP, its Default Gateway field initially showed 0.0.0.0. As a result, the laptop could not reach gateways of other VLANs, even though it could obtain an address on its own subnet.

The cause of the issue was that the DHCP pool ADMIN_IT_POOL did not initially contain the default-router statement, so the router was not advertising a default gateway to clients in that pool.

The issue was corrected by adding the missing default-router command to the ADMIN_IT_POOL configuration:

configure terminal ip dhcp pool ADMIN_IT_POOL default-router 192.168.1.1 exit end

After this correction, the client renewed its DHCP configuration and received the correct default gateway address of 192.168.1.1, which restored inter-VLAN communication from the Admin/IT wireless client to the rest of the network.

# 21. CONCLUSION

The XYZ Company Savar Branch network was successfully designed and implemented in Cisco Packet Tracer. The final network provides department-based VLAN segmentation for Admin/IT, Finance/HR and Customer Service, along with a separate native/management VLAN.

Both wired and wireless connectivity were implemented for all departments, with automatic DHCP addressing configured for each department VLAN. Inter-VLAN routing was achieved using the Router-on-a-Stick method, and WPA2 wireless security was applied to all access points. Communication testing using ping confirmed successful connectivity within and between VLANs.

It should be noted that no Internet or WAN connection was configured in this project, and the network operates as a self-contained local area network for the Savar branch.

# 22. REFERENCES

-  Cisco Networking Academy / Cisco Packet Tracer documentation.

-  Cisco IOS command reference documentation.

-  Course lecture materials.

# APPENDIX A: COMPLETE SWITCH CONFIGURATION

enable configure terminal hostname SW-SAVAR vlan 10 name ADMIN_IT exit vlan 20 name FINANCE_HR exit vlan 30 name CUSTSERVICE exit vlan 99 name MGMT_NATIVE exit interface gigabitEthernet 0/1 switchport mode trunk switchport trunk native vlan 99 exit interface range fastEthernet 0/2 - 3 switchport mode access switchport access vlan 10 exit interface range fastEthernet 0/4 - 5 switchport mode access switchport access vlan 20 exit interface range fastEthernet 0/6 - 7 switchport mode access switchport access vlan 30 exit interface fastEthernet 0/10 switchport mode access switchport access vlan 10 exit interface fastEthernet 0/11 switchport mode access switchport access vlan 20 exit interface fastEthernet 0/12 switchport mode access switchport access vlan 30 exit end copy running-config startup-config

# APPENDIX B: COMPLETE FINAL ROUTER CONFIGURATION

enable configure terminal hostname R-SAVAR interface gigabitEthernet 0/0 no shutdown exit interface gigabitEthernet 0/0.10 encapsulation dot1Q 10 ip address 192.168.1.1 255.255.255.192 exit interface gigabitEthernet 0/0.20 encapsulation dot1Q 20 ip address 192.168.1.65 255.255.255.192 exit interface gigabitEthernet 0/0.30 encapsulation dot1Q 30 ip address 192.168.1.129 255.255.255.192 exit interface gigabitEthernet 0/0.99 encapsulation dot1Q 99 native ip address 192.168.1.193 255.255.255.192 exit ip dhcp excluded-address 192.168.1.1 192.168.1.10 ip dhcp excluded-address 192.168.1.65 192.168.1.75 ip dhcp excluded-address 192.168.1.129 192.168.1.139 ip dhcp pool ADMIN_IT_POOL network 192.168.1.0 255.255.255.192 default-router 192.168.1.1 dns-server 8.8.8.8 exit ip dhcp pool FINANCE_HR_POOL network 192.168.1.64 255.255.255.192 default-router 192.168.1.65 dns-server 8.8.8.8 exit ip dhcp pool CUSTSERVICE_POOL network 192.168.1.128 255.255.255.192 default-router 192.168.1.129 dns-server 8.8.8.8 exit end copy running-config startup-config

# APPENDIX C: VERIFICATION COMMANDS

The following commands were used to verify the network configuration.

## Switch

show vlan brief show interfaces trunk show interfaces gigabitEthernet 0/1 switchport

show vlan brief verifies VLAN creation and port membership. show interfaces trunk verifies trunk status and allowed VLANs. show interfaces gigabitEthernet 0/1 switchport verifies the switchport mode and native VLAN of the trunk interface.

## Router

show ip interface brief show ip dhcp pool show ip dhcp binding

show ip interface brief verifies the status of each subinterface. show ip dhcp pool displays the configuration of each DHCP pool. show ip dhcp binding verifies the IP addresses leased to client devices.

## Client

ipconfig ping 192.168.1.1 ping 192.168.1.65 ping 192.168.1.129

ipconfig displays the IP configuration of the client. The ping commands verify reachability of the gateway addresses of each department VLAN from a client device.