3. Configuration Commands (Cisco CLI)

Step 1: Initialize Database and Create VLANs

Execute these commands on Switch0, Switch1, and Switch2:

text

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Faculty

Switch(config-vlan)# exit

Switch(config)# vlan 11

Switch(config-vlan)# name Students

Switch(config-vlan)# exit

Switch(config)# vlan 12

Switch(config-vlan)# name Guest

Switch(config-vlan)# exit

Step 2: Configure Access Ports for End Devices

Assign the physical switch interfaces to their designated logical networks.

On Switch0:

text

Switch0(config)# interface range fa0/1-2

Switch0(config-if-range)# switchport mode access

Switch0(config-if-range)# switchport access vlan 10

Switch0(config-if-range)# exit

Switch0(config)# interface range fa0/3-4

Switch0(config-if-range)# switchport mode access

Switch0(config-if-range)# switchport access vlan 11

Switch0(config-if-range)# exit

Switch0(config)# interface range fa0/5-6

Switch0(config-if-range)# switchport mode access

Switch0(config-if-range)# switchport access vlan 12

Switch0(config-if-range)# exit

(Repeat corresponding range mappings on Switch1 and Switch2 based on your specific topology layout port-bindings).

Step 3: Configure Trunk Links Between Switches

Inter-switch connections must carry traffic for all three VLANs simultaneously.

On Switch0 (Interfaces connecting to Switch1 and Switch2):

text

Switch0(config)# interface range fa0/23-24

Switch0(config-if-range)# switchport mode trunk

Switch0(config-if-range)# switchport trunk allowed vlan 10,11,12

Switch0(config-if-range)# no shutdown

(Apply identical trunk commands on the corresponding interconnecting interfaces of Switch1 and Switch2).

4. Verification and Testing

VLAN Database Verification

To ensure VLANs are correctly created and mapped to physical ports, execute:

text

Switch# show vlan brief

- Expected Output: VLAN 10, 11, and 12 must display an status of active alongside their assigned FastEthernet ports.

Trunking Link Verification

To verify operational inter-switch trunk links, execute:

text

Switch# show interfaces trunk

- Expected Output: The interconnected ports should list their mode as on, status as trunking, and native VLAN as 1.

ICMP Connectivity Testing (Ping)

- Intra-VLAN Test: A ping from PC0 (VLAN 10) to PC5 (VLAN 10) must return a Successful (Reply) status.

- Inter-VLAN Test: A ping from PC0 (VLAN 10) to PC1 (VLAN 11) must return a Failed (Timed Out) status. This confirms logical traffic isolation works perfectly without a Layer 3 routing device.

- 5. Conclusion

- The lab successfully demonstrates network segmentation across three separate switches using Cisco Packet Tracer. Broadcast domains were localized by limiting VLAN traffic strictly to assigned access ports and traversing inter-switch paths via 802.1Q tagged trunks.